MODULAR BATTERY MANAGEMENT SYSTEM
Technical Report

ESP32 • Wokwi Simulation • Blynk IoT

Prepared by: NYALAKNTI SRISHANTH
Department: ELECTRONICS AND COMMUNICATION ENGINEERING
Submission date: 11/10/2026

# Abstract

The Modular Battery Management System (BMS) is designed to monitor individual battery-cell voltages, assess battery operating conditions, identify abnormal states, and provide real-time monitoring through Internet of Things (IoT) connectivity. The system utilizes an ESP32 microcontroller and a CD74HC4067 analog multiplexer to acquire voltage measurements from multiple simulated battery cells, supporting a configurable cell range of 4 to 16 cells. It evaluates cell-voltage imbalance by identifying the highest and lowest cell voltages, estimates the State of Charge (SoC), and implements adaptive threshold-based decision logic using SoC and current measurements to support battery condition assessment and relay control. Additional features include fault-status monitoring, system-state management, visual and audible alerts, an LCD interface, and remote telemetry through the Blynk IoT platform. The prototype was developed and evaluated in the Wokwi simulation environment to demonstrate its monitoring capabilities and system integration. Simulation testing demonstrated the system’s ability to monitor active cells across the configured range and establish successful Wi-Fi and Blynk connectivity. The project integrates embedded-system design, battery monitoring algorithms, adaptive decision-making, fault-status reporting, and IoT-based visualization into a modular BMS prototype. The developed system provides a foundation for further validation, sensor calibration, and future implementation with physical battery hardware.

# INTRODUCTION

A Battery Management System (BMS) is an essential component of modern battery-powered applications, including electric vehicles, renewable energy storage systems, portable electronic devices, and industrial equipment. Its primary purpose is to monitor battery operating conditions, evaluate cell performance, identify abnormal conditions, and support the safe and efficient operation of a battery pack. In a battery pack consisting of multiple cells, differences in individual cell voltages can develop during charging and discharging. Monitoring these differences is important because excessive cell-voltage imbalance can affect battery performance, usable capacity, and operating reliability.

The Modular Battery Management System (BMS) project is designed to demonstrate battery-cell monitoring, voltage-imbalance assessment, fault detection, and Internet of Things (IoT)-based monitoring using an ESP32 microcontroller. A CD74HC4067 analog multiplexer is used to select and measure multiple simulated cell-voltage inputs through the ESP32 analog-to-digital converter (ADC). The firmware processes the acquired measurements to identify the strongest and weakest cells, calculate voltage imbalance, and estimate the State of Charge (SoC) using a simplified voltage-based method.

An adaptive threshold mechanism is incorporated to adjust the imbalance threshold according to the estimated SoC and measured current. The system also includes state-management logic to represent normal, fault, and recovery conditions. A relay output, three status LEDs, a buzzer, and an LCD provide local indications of system operation and abnormal conditions. These features help demonstrate how measurement data can be used to support monitoring and control decisions within a battery-management application.

To enable remote monitoring, the project integrates the ESP32 with the Blynk IoT platform through Wi-Fi connectivity. Selected parameters, including cell-voltage imbalance, strongest and weakest cell voltages, relay status, fault state, SoC, Wi-Fi signal strength, and risk score, can be displayed on a remote dashboard. This integration combines embedded-system programming, sensor-data processing, decision-making logic, and IoT communication within a single prototype.

The project is developed and tested in the Wokwi simulation environment, which allows the circuit and firmware behavior to be evaluated without requiring a physical battery pack. Simulation-based testing helps demonstrate the interaction between the sensing interface, processing algorithms, state-management logic, local indicators, and remote telemetry. However, the voltage and current conversions are simplified, and the estimated SoC is not a calibrated battery measurement. Therefore, the prototype demonstrates the intended monitoring concepts rather than establishing the safety or performance of a real battery system.

Components:

| - HARDWARE COMPONENTS |  |
| --- | --- |
| - COMPONENT | - PURPOSE AND QUANTITY |
| - ESP32 | - Acts as the main controller, processes sensor readings, executes BMS logic, and provides Wi-Fi connectivity. (1) |
| - CD74HC4067 16-Channel Analog Multiplexer | - Selects individual analog input channels to monitor multiple simulated battery cells using a shared analog input. (1) |
| - Potentiometers | - Simulate battery-cell voltage variations for testing cell monitoring and imbalance detection. (4-16) |
| - Current-Sensing Input (Potentiometer) | - Provides an analog signal used to estimate the battery discharge current. (1) |
| - 16×2 I²C LCD Display | - Displays battery parameters and system status locally. (1) |
| - LED’S (RED, YELLOW, GREEN) | - Indicates the normal operating status, according to the implemented firmware logic. (3) |
| - Buzzer | - Provides an audible alert when activated by the control logic. (1) |
| - Relay Module | - Represents a switching mechanism controlled by the battery-monitoring logic. (1) |
| - Connecting Wires | - Establish electrical connections between the ESP32 and the peripheral components. |

| - Software and development Tools |  |
| --- | --- |
| - TOOLS | - PURPOSE AND QUANTITY |
| - Wokwi Simulator | - Provides a virtual environment for developing and testing the circuit without a complete physical hardware setup. |
| - Blynk IoT Platform | Enables remote monitoring of selected battery parameters and system status over Wi-Fi. |
| - ESP32 Wi-Fi Library | - Establishes wireless network connectivity. |
| - Blynk Library | - Handles communication between the ESP32 firmware and the Blynk IoT platform. |
| - I²C LCD Library | Supports communication with and control of the LCD display. |
| - Serial Monitor | Displays diagnostic messages, cell-voltage readings, calculated parameters, and system status during testing. |

# Architecture

Simulated Battery Cells  (4-16)

Potentiometers representing individual cell voltages

CD74HC4067 Analog Multiplexer

Selects individual cell-voltages inputs

ESP32 Microcontroller

Data acquisition | Voltage calculations | SoC estimation | Imbalance analysis

Adaptive threshold logic | Fault monitoring | System-state management

OUTPUT  Control

Relay, LEDs & Buzzer

Local Display

LCD and serial monitor

Wi-Fi Communication

ESP32 connects to network and sends selected telemetry

Blynk IoT  Dashboard

Remote visualization of voltage imbalance, cell voltages, Soc, relay status, fault status, system state, WiFi RSSI & risk score

### Description of Architecture Modules

1. Battery Cell Input Module

This module represents individual battery cells using potentiometers in the Wokwi simulation. The input values can be varied to simulate differences in cell voltage and evaluate the system's monitoring and imbalance-detection capabilities.

2. Data Acquisition Module

The CD74HC4067 analog multiplexer selects individual cell-voltage inputs and routes the selected signal to the ESP32 analog input. The ESP32 reads the signal through its ADC and converts the raw readings into estimated cell-voltage values using the configured conversion formula.

3. Processing and Monitoring Module

The ESP32 processes the acquired measurements to identify the strongest and weakest cells and calculate voltage imbalance. It also estimates SoC from cell-voltage measurements and processes the current-related input to support battery-condition assessment.

4. Decision and Fault Monitoring Module

The firmware evaluates the calculated parameters against configured thresholds. Adaptive threshold logic considers SoC and current measurements, while the system-state and fault-monitoring logic manage normal, fault, recovery, and other configured operating conditions. The resulting status information is used to support output control and monitoring.

5. Output and Display Module

The relay output, LEDs, and buzzer provide control and indication functions according to the implemented firmware logic. The LCD and Serial Monitor display cell measurements, battery parameters, system states, and diagnostic information.

6. IoT Communication Module

The ESP32 uses Wi-Fi to communicate with the Blynk IoT platform. Selected parameters are transmitted to the dashboard, allowing users to remotely observe battery-monitoring data, relay status, fault information, and other telemetry.

7. Simulation and Testing Module

The Wokwi environment provides the virtual circuit in which the ESP32, multiplexer, simulated cell inputs, display, and output devices can be evaluated. This allows the firmware logic and component interactions to be tested before physical hardware implementation.

# HARDWARE CONNECTIONS

ESP32 and CD74HC4067 Connections:

| CD74HC4067 pin | ESP32 connection | Function |
| --- | --- | --- |
| SIG / COM | GPIO 35 | Multiplexer analog output to ESP32 ADC |
| S0 | GPIO 26 | Channel-selection bit 0 |
| S1 | GPIO 25 | Channel-selection bit 1 |
| S2 | GPIO 33 | Channel-selection bit 2 |
| S3 | GPIO 32 | Channel-selection bit 3 |
| VCC | 3.3 V | Multiplexer power supply, subject to module specifications |
| GND | GND | Common ground |
| EN | GND | Enables the multiplexer |

Simulated Battery-Cell Connection:

| Simulated cells | Multiplexer channel | Input connection |
| --- | --- | --- |
| CELLS(4-16) | C0-C15 | Potentiometer Output |

## Current Input, Relay, LEDs, and Buzzer:

| Component | ESP32 GPIO | Connection / purpose |
| --- | --- | --- |
| Current-sensing analog output | GPIO 34 | Provides the analog current-related signal |
| Relay module IN | GPIO 15 | Controls relay switching |
| Green LED | GPIO 17 | Indicates normal fault state |
| Yellow LED | GPIO 16 | Indicates degraded fault state |
| Red LED | GPIO 4 | Indicates failsafe or shutdown fault state |
| Buzzer signal | GPIO 13 | Provides audible alerts based on fault severity |

LCD Display Connections

| LCD pin | ESP32 connection | Function |
| --- | --- | --- |
| VCC | Suitable supply for the LCD module | Power |
| GND | GND | Common ground |
| SDA | GPIO 21 (typical ESP32 default) | I²C data |
| SCL | GPIO 22 (typical ESP32 default) | I²C clock |

Software Configurations:

Wi-Fi and Blynk IoT Connections:

| ## Parameter | ## Configuration |
| --- | --- |
| ## Wi-Fi network | ## Wokwi-GUEST |
| ## Wi-Fi password | ## Empty |
| ## Wi-Fi channel | ## 6 |
| ## IoT platform | ## Blynk IoT |
| ## Communication method | ## Wi-Fi |
| ## Blynk virtual pins | ## V0–V9 |

## CODE STRUCTURE

| Module | Purpose |
| --- | --- |
| Library inclusion and configuration | Defines libraries, GPIO pins, constants, and Blynk settings. |
| Initialization (setup()) | Initializes serial communication, GPIO, LCD, Wi-Fi, and Blynk. |
| Sensor acquisition | Reads analog inputs and selects multiplexer channels. |
| Voltage processing | Converts ADC readings into estimated cell voltages. |
| Battery analysis | Calculates imbalance, identifies strongest and weakest cells, and estimates SoC. |
| Decision logic | Evaluates adaptive thresholds and relay conditions. |
| Fault management | Classifies system conditions and manages recovery. |
| Output management | Updates the LCD, LEDs, buzzer, and relay. |
| Telemetry | Sends selected measurements and status information to Blynk. |
| Main execution (loop()) | Repeatedly executes monitoring and control functions. |

The exact function names and execution order should be checked against the final source code submitted with the project.

## Sensor Acquisition and Multiplexer Algorithm

The CD74HC4067 multiplexer allows the ESP32 to read multiple simulated battery-cell inputs using one ADC input, GPIO 35.

### Working principle

  - The ESP32 sets select pins S0–S3 to choose a multiplexer channel.

  - The selected channel is connected to the SIG output.

  - The ESP32 reads the analog signal through GPIO 35.

  - The firmware discards the first ADC sample after switching channels to reduce signal-settling errors.

  - The ADC value is converted into an estimated cell voltage.

  - The process repeats for each configured cell.

For a 16-channel multiplexer, the channel number is represented by four binary select bits. For example, channel 5 uses the binary pattern 0101; verify bit-to-pin order against the circuit.

### Voltage conversion

V_cell = (ADC reading / 4095) * 4.2

ADC reading is the digital value obtained from the ESP32 ADC; 4095 is the maximum value for a 12-bit ADC reading;

4.2 V is the assumed full-scale cell-voltage range used by the firmware.

## Cell-Voltage Imbalance Algorithm

The algorithm calculates the difference between the highest and lowest measured cell voltages.

Steps: read the configured cell voltages, identify the highest and lowest voltages, subtract the lowest from the highest, and convert volts to millivolts.

V_imbalance = (V_max - V_min) * 1000

Example: strongest cell = 3.671 V; weakest cell = 3.609 V.

(3.671 - 3.609) * 1000 = 62 mV

The result is approximately 62 mV, depending on rounding. Comparing the current imbalance with the previous value can indicate whether imbalance is increasing, decreasing, or stable.

## State of Charge (SoC) Estimation

The firmware estimates SoC from the average measured cell voltage.

V_avg = (sum of all cell voltages) / N SoC(%) = ((V_avg - 3.0) /(4.2 - 3.0)) * 100

| Average voltage | Estimated SoC |
| --- | --- |
| 3.0 V | 0% |
| 3.3 V | 25% |
| 3.6 V | 50% |
| 3.9 V | 75% |
| 4.2 V | 100% |

The result is limited to the 0–100% range. This is a simplified voltage-based estimate. Actual SoC depends on chemistry, load, temperature, cell condition, and voltage relaxation; this formula is not a precision fuel gauge.

## Adaptive Threshold Algorithm

The adaptive threshold changes according to estimated SoC and measured current.

| Condition | Base threshold |
| --- | --- |
| SoC greater than 80% | 30 mV |
| SoC below 20% | 20 mV |
| SoC from 20% to 80% | 50 mV |
| Current greater than 1 A | Add 15 mV |

If current exceeds 1 A, 15 mV is added to the base threshold. Example: at SoC = 52.9% and current = 2.16 A, the base threshold is 50 mV and the current adjustment is 15 mV.

T_adaptive = 50 + 15 = 65 mV

## Relay Control Algorithm

- The relay uses threshold comparison and hysteresis to reduce rapid switching.

- Switch ON: when imbalance exceeds the greater of the adaptive threshold and 60 mV.

- Switch OFF: when imbalance falls below 40 mV.

- Safety override: force relay OFF in FAILSAFE or SHUTDOWN.

- Debouncing: a 300 ms debounce interval is used.

T_ON = max(T_adaptive, 60 mV)

| Imbalance / condition | Expected behavior |
| --- | --- |
| Below 40 mV | An ON relay can switch OFF. |
| 40–65 mV (for a 65 mV ON threshold) | Retain the previous state. |
| Above 65 mV | An OFF relay can switch ON. |
| FAILSAFE or SHUTDOWN | Force the relay OFF. |

The intermediate region is the hysteresis band. These rules describe firmware logic; they do not establish real battery isolation or balancing without suitable physical circuitry.

## Fault Detection Algorithm

The documented checks include: cell voltage outside the expected 3.0–4.2 V range; severe low-cell voltage below

2.5 V; sudden voltage changes greater than 1.0 V between samples; a potentially frozen reading over approximately 20 samples while another cell changes by more than 0.05 V; and severe cell-voltage imbalance greater than 500 mV.

These checks identify potential measurement or operating problems; they are not a fully validated hardware protection system.

| Fault state | Description | Indication |
| --- | --- | --- |
| NORMAL | No active fault condition | Green LED |
| DEGRADED | A fault or abnormal condition is detected | Yellow LED and slower buzzer pattern |
| FAILSAFE | A severe condition requires protective behavior | Red LED and faster buzzer pattern |
| SHUTDOWN | Defined as a fault state in the firmware | Red LED and faster buzzer pattern |

The firmware includes a relay-off override for FAILSAFE and SHUTDOWN. The current implementation does not automatically enter SHUTDOWN through its transition logic, and relay-mismatch detection is not implemented as a real feedback check.

## System State Machine and Recovery

The system-state machine uses three states: NORMAL (no active condition requiring fault handling), FAULT (an abnormal condition is detected and the appropriate response is applied), and RECOVERY (the system checks whether the fault has cleared).

The firmware uses a 5-second recovery verification period. Return to normal operation depends on the implemented transition rules. The fault-state machine separately tracks NORMAL, DEGRADED, FAILSAFE, and SHUTDOWN.

## LCD, LED, and Buzzer Control

The LCD displays cell measurements, imbalance, system state, and telemetry information. The green LED indicates normal operation; the yellow LED indicates a degraded condition; the red LED indicates a failsafe or shutdown condition. The buzzer generates alert patterns depending on fault state, and the relay follows imbalance threshold logic and safety overrides.

The documented LCD refresh interval is approximately 250 ms. Displayed content depends on the current page and any critical-fault override.

Wi-Fi, Blynk, and Telemetry

The ESP32 uses the Wokwi-GUEST network in the simulation. When connected, the firmware updates Blynk virtual datastreams.

| Virtual pin | Data transmitted |
| --- | --- |
| V0 | Cell-voltage imbalance (mV) |
| V1 | Strongest cell voltage (V) |
| V2 | Weakest cell voltage (V) |
| V3 | Relay state |
| V4 | Fault state |
| V5 | Wi-Fi RSSI (dBm) |
| V6 | Telemetry queue depth |
| V7 | System state |
| V8 | Estimated SoC (%) |
| V9 | Risk score (0–100) |

The documented telemetry queue capacity is 50 events. Events may contain a timestamp, imbalance, SoC, current, relay status, fault state, and event type. The documented telemetry transmission interval is approximately 3 seconds. The queue provides temporary event storage but does not guarantee permanent retention or successful delivery during a prolonged disconnection.

.

Instructions for Running the Wokwi Simulation

Wokwi is an online electronics simulator used to test the Modular BMS without initially assembling physical hardware. The simulation

represents the ESP32, CD74HC4067 multiplexer, simulated cell-voltage inputs, current input, LCD, LEDs, buzzer, and relay output

.

Requirements:

• A web browser with internet access.

• The BMS Wokwi project containing the circuit and firmware.

• An ESP32 development board selected in the simulation.

• Required Arduino libraries.

• A valid Blynk template and device configuration if remote monitoring is enabled.

Steps to run the simulation:

Step 1 — Open the project. Open the existing Modular BMS project in Wokwi. Confirm the ESP32, multiplexer, cell-input controls, LCD,

indicators, buzzer, and relay are present.

Step 2 — Check firmware and wiring. Verify GPIO assignments. Multiplexer SIG should connect to GPIO 35, select pins to GPIO 26,

25, 33, and 32, and current input to GPIO 34.

Step 3 — Start simulation. Press the green Start Simulation button. Wait for compilation and initialization. If compilation fails, review the

error and correct code or library configuration.

Step 4 — Adjust simulated cell voltages. Change potentiometer controls for cell inputs. Observe readings in Serial Monitor, LCD, or

dashboard. Keep inputs within the intended range during normal testing.

Step 5 — Monitor output. Check cell voltages, strongest and weakest cells, imbalance, estimated SoC, current, relay state, and fault

status in Serial Monitor.

Step 6 — Verify Blynk connectivity. If enabled, check Wi-Fi and Blynk connection. Confirm dashboard widgets correspond to virtual

pins V0–V9.